ISSN  1999-8716

Diyala Journal

of Engineering Sciences

Printed in Iraq

Vol. 03 , No. 01, pp. 16-24 , June 2010

SIMULATION OF RECTANGULAR MICROSTRIP ANTENNA BY INTEGRATING MATLAB IN VISUAL BASIC

Hussain Falih Mahdi

Engineering College , Diyala University

(Received:17-5-2009 ; Accepted:16-11-2009)

ABSTRACT - Microstrip antenna has many applications nowadays. The rectangular microstrip antenna is one of the microstrip antenna types, it can be designed using many softwares. In this paper, a new method was suggested to do the simulation; by integrating Matlab in Visual Basic. Due to the good development of the personal computers, the simulations methods were activated and used widely. No software has the visual window that meets some of our requirements. It is well known that the visual basic is not completely a scientific language, thus, by adding to it a strong scientific ability, it will be very powerful language. This step was done successfully and the rectangular microstrip antenna was simulated in this way.

KEY WORDS:- matlab, visual basic, microstrip antenna, antenna simulation, matlab simulation program, com builder.

1- INTRODUCTION

In high-performance aircraft, spacecraft, satellite, and missile applications, where size, weight, cost, performance, ease of installation, and aerodynamic profile are constraints, and low-profile antennas may be required. Presently there are many other government and commercial applications, such as mobile radio and wireless communications that have similar specifications. To meet these requirements, microstrip antennas can be used. Some of the students that need to do a simulation for a microstrip antenna prefer to do it using visual basic, but when he start; his programming, he will face many complex problems that cannot be programmed simply in visual basic, even so, he needs at least of about 1500 code line to do a very simple simulation. At this point and without dispute he will leave the programming in