ISSN  1999-8716

Diyala Journal

of Engineering Sciences

Printed in Iraq

Vol. 03 , No. 01 , pp. 47-63 , June 2010

STUDYING THE STRESS ANALYSIS IN LEAF SPRING BY FINITE ELEMENTS METHOD

Abdul Karim Selman Abdul Karim 1, and Arshed Abdul Hamed Mohammed2

1University of technology

2College of engineering , Diyala University

(Received:30-5-2009 ; Accepted:12-1-2010)

ABSTRACT - Light truck is considerably crucial for all the world economic growth since it was used in transportation for various kinds of commodities. Leaf spring is the most admire alternative utilized in the suspension system of this specific truck due to their low production cost, reliability and maintainability. In this, study a rational technique to design leaf spring for light truck is investigated. Simulation resembles to constrained condition when mounting on the vehicles were undertaken to examine the stresses distribution for each leaf. In this research the leaf spring is used on light truck with following  information:

Number of leaves equal five, Thickness of each leaf equal 10mm, length of distance from vertical line that divides the leaf spring to two equal parts, to one end of side of leaf spring, varies from (500 to 1000)mm, Radius of bow- shaped leaf spring varies from (1250 to 3000)mm and constant load applied on the first leaf equal 1000N on each side. The number of cases studied equal 24 case.

The finite element method was applied as a method of analysis to examine the stress distribution for each leaf.

KEY WORDS:- stress  leaf  spring  finite  element.

1- INTRODUCTION

In order to provide satisfactory comfort, shocks from bumps in the road surface must not be transmitted to the cab and superstructure. Springs are therefore provided between wheels and the chassis frame.