ISSN  1999-8716

Diyala Journal

of Engineering Sciences

Printed in Iraq

Vol. 03 , No. 02 , pp. 80-96 , December 2010

CALCULATION OF RELATIVE EXTRUSION PRESSURE FOR CIRCULAR SECTION BY LOCAL COORDINATES SYSTEM BY USING FINITE ELEMENT METHOD F.E.M

Jabbar  Kasim  Jabbar

Engineering College , Diyala University.

(Received:18-1-2010; Accepted:30-6-2010)

ABSTRACT -  Large development that happens in computers technology leads to the use of modern programs and pollutant about different forming processes from through design and analysis for geometric forms by using Finite Element Method F.E.M, which gives accurate results which approaches experimental results. This study depends on local coordinates system by using rotation of local coordinates for nodes on region contact length between metal and die. This is to calculate relative extrusion pressure and study the  effect of relative die length  on the relative extrusion pressure for reduction of area (20%, 40%, 60%, 80%) with friction factor (0.1) by using software ANSYS program. The contours for distribution of stresses and plastic strains were obtained, to obtain perfect die length that is needed to less extrusion pressure.

The results of this study are compared with experimental result and upper bound theory and have shown a good agreement with a minimum discrepancy.

Keyword:- Relative Extrusion Pressure, Local Coordinates System, Finite Element Method.

Introduction

The extrusion process is an attractive production method in industry for its ability to achieve energy and material saving, quality improvement and development of homogenous properties throughout the component (1). Extrusion process is one of the important forming processes where put mass of metal in container and it is pushed by using external load which makes the substance flow from a hole which represents shape of product (2) , extrusion processes are divided into two types depending on the direction of motion for metal, the first